BRISTOL

BRITANNIA

Series 253

Cockpit Manual

Version 4.0 April 2007

For use with Mike Stone’s Britannia aircraft for Flight Simulator 2004

by Fraser A. McKay

Britannia 253

CONTENTS

MUST READ

- ELECTRICAL SYSTEM

- ENGINE & PROPELLER CONTROLS

- FUEL SYSTEM

- FLYING CONTROLS, FLAPS & UNDERCARRIAGE

- DEICING SYSTEMS

- MINOR SYSTEMS

- FLIGHT SYSTEM, AUTOPILOT & RADIOS

- LIMITATIONS

- ENGINE & FLIGHT HANDLING

MUST READ

The Britannia is probably the most complex airliner of its generation, requiring a minimum of three crew. Most of its systems have been replicated here, within the limitations of Flight simulator.

While the panel is designed for the serious enthusiast, the casual user can get away with

operating the aircraft, so long as the systems are left alone. The aircraft will load with the systems in a takeoff configuration.

The engine and propeller controls are fully governed by the panel, therefore keyboard and joystick propeller and mixture controls should not be used.

The controls appear in a number of windows, opened by their respective icons on the main panel, and all can be closed by clicking on the cross on the top right of each panel.

The cockpit fenestration on the Britannia is less than optimal for FS use, having a large number of awkwardly placed pillars. The pillar directly in front of the pilot may be moved to the left by clicking on it to improve the runway view.

Britannia 253

1. ELECTRICAL SYSTEM

The controls pertaining to the electrical system are mainly grouped on the starboard systems panel which is accessed with the  icon. The casual user can avoid altering most of the controls, however the electrical window must be opened for the systems to take effect. The window may then be closed again.

Main Supplies

Each engine drives an alternator which will only come on line when the respective propeller RPM (PRPM) is approximately 550 or greater. The alternator will drop out if the PRPM is allowed to fall below this value. A red MAIN POWER FAILURE warning lamp on the main instrument panel will light when any alternator which is not switched off drops off line.

The output from each alternator is split into three parts. The first part is a 200VAC supply which is used as follows:

No1 Alternator---200VAC output supplies No1 PH Busbar, which supplies the propeller

deicing, and No1 hydraulic pump.

No2 Alternator---200VAC output as used as as standby as well as supplying No2 hydraulic

pump.

No3 Alternator---200VAC output supplies No3 PH Busbar, which supplies the continuous

tail deicing and No3 hydraulic pump.

No4 Alternator---200VAC output supplies No4 PH Busbar, which supplies the cyclic tail

deicing and No4 hydraulic pump.

Britannia 253

Each alternator has a three position double throw control switch, ON, OFF and RESET,

an overvolts magnetic indicator, a red failure warning lamp and an earth leakage magnetic

indicator.  Additionally Nos1,3,and 4 alternators have an isolation switch adjacent to the control switch which allows an alternative alternator to power the respective bus. Two alternators cannot be connected to the same bus at once, however an alternator can supply two busses. The selections which can be made are:

No1 Isolation Switch---ISOLATE--- No2 Alternator supplies No1 PH Busbar

No1 Isolation Switch---C/OVER ---  No4 Alternator supplies No1 PH Busbar

No3 Isolation Switch---ISOLATE--- No2 Alternator supplies No3 PH Busbar

No3 Isolation Switch---C/OVER ---  No4 Alternator supplies No3 PH Busbar

No4 Isolation Switch---ISOLATE--- No2 Alternator supplies No4 PH Busbar

No4 Isolation Switch---C/OVER ---  No1 Alternator supplies No4 PH Busbar

The failed alternator(s) should first be switched off.

The second part of the alternator output is rectified and supplies the 112VDC Medium Voltage (MV) system Busbars A and B. The busbars are normally connected via contactors and MV Bus C which is in turn connected to four 24V batteries in series to provide a 112V backup supply. Magnetic indicators show continuity when the busses are connected, however the busses can be split by operating momentarily the Regain Generation handle at the forward end of the roof panel (seen in the main view). A MV master rotary switch has three positions; OFF, FLIGHT and GROUND. With the switch set at Ground all services are available, provided there is sufficient battery power, and the green lamp will light and the magnetic indicator shoes continuity.

Summary of systems supplied by the MV system:

MV Bus A: Propeller Feathering Nos1 and 4        Intake Heater Mats Nos1 and 4

Flap Motors      Fuel Booster Pumps

MV Bus B: Propeller Feathering Nos2 and 3        Intake Heater Mats Nos 3 and 2

Fuel Booster Pumps

MV Bus C: Propeller Standby controls                  Engine Starter Motors

Note that while these systems operate on the 112VDC MV supply, their controls and indicators operate on 28VDC.

The third part of the alternator output is rectified and reduced in voltage and supplies the

28VDC Low Voltage (LV) system busbars A and B. The two busbars are normally connected as in the MV system and can be split in the same way. Two 24V batteries are connected in parallel to the LV system. The LV system also has a master rotary switch with the same functions as the MV system.

All of the remaining DC systems operate on 28VDC.

Note that there is no FS master switch as such, however, switching the MV and LV master switches off will have the same effect.

Britannia 253

Secondary Supplies

The secondary electrical system operates at 115VAC and is provided by four normal inverters and a standby inverter. Nos 1 and 2 inverters as well as the standby take their supply from the MV Bus A and Nos 3 and 4 inverters from MV Bus B. The normal inverters

feed their individual busbars, Secondary A, B, C and D. A red SECONDARY POWER FAILURE warning lamp is on the main instrument panel.

Each normal inverter is controlled by a three position double throw switch, ON, 1ST TRANSFER and 2ND TRANSFER. When a switch is moved to the 1ST TRANSFER position, the standby inverter, if it is live, takes the load of that particular inverter. With the switch at 2ND TRANSFER the load is taken as follows:

No1 Inverter---2ND TRANSFER--- Load taken by No4 Inverter (Autopilot shed)

No2 Inverter---2ND TRANSFER--- Load taken by No3 Inverter

No3 Inverter---2ND TRANSFER--- Load taken by No2 Inverter

No4 Inverter---2ND TRANSFER--- Load taken by No1 Inverter

Magnetic indicators show continuity when the respective busses are powered. A single voltmeter and rotary selector switch enable the voltage on each bus to be checked.

Summary of services supplied by the Secondary system:

Secondary Bus A: No1 Normal Propeller Controller           No1 ADF

No1 Flowmeter                                      Standby Attitude

Port Fuel Contents                                Servo Altimeter

No1 ADF                                               No1 Oil Pressure

Autopilot                                                No1 VOR/ILS

Secondary Bus B: No2 Normal Propeller Controller           SFS

No2 Flowmeter                                      Nos 1 & 4 CRPM sensitive scale

No2 Oil Pressure

Secondary Bus C: No3 Normal Propeller Controller           No2 ADF

No3 Flowmeter                                      Centre & Stbd Fuel Contents

No3 Oil Pressure                                   No2 VOR/ILS

Secondary Bus D: No4 Normal Propeller Controller           No2 ADF

No4 Flowmeter                                      Nos 2 & 3 CRPM sensitive scale

No4 Oil Pressure                                   RMI

The standby inverter should be on during takeoff and landing, and switched off in flight if not required.

Britannia 253

2. ENGINE & PROPELLER CONTROLS

Throttles and Propeller Controls

Unlike most of its contemporaries, the Britannia has independent controls for the throttle and propeller. The throttle opening is electrically controlled via the throttle lever, there being no mechanical linkage between the two. The quadrant is accessed with the  icon.

Lever movement is restricted with the gust locks engaged, preventing takeoff power being selected.

The propellers are controlled electrically from the control panel opened with the  icon.

By default the propellers are at the maximum PRPM setting in flight fine pitch, ready for takeoff.

Each propeller is controlled by its own switch, spring loaded to the centre off position. The switches are held forward to increase PRPM and back to decrease. A magnetic indicator shows MAX when maximum PRPM is selected and MIN when minimum is selected. The indicators show black at other times. Additionally a collective switch between the individual switches can be used to adjust the propellers together. The actuators take approximately 12 seconds to run from MAX to MIN. The propeller controls will only function with the Master Propeller switch OFF

The Master Propeller switch in the centre of the panel has three positions, centre OFF, and is used to select the synchroniser on and either No1 or No2 propeller as the master to which the others are synchronised. The synchroniser cuts out if PRPM falls below 760.

Britannia 253

A standby propeller control is provided, and comprises a master rotary switch guarded by a black and yellow painted cover, which can be lifted by clicking in its finger lug, and a single increase/decrease switch. The propeller is selected on the rotary switch, then the standby switch can be used to alter the PRPM of that propeller. The rotary switch should be set OFF when not in use. With the switch at lock, the actuators are electrically locked and both the normal and standby systems are inoperative. Note that the standby system can operate the propeller from MAX all the way to FEATHER, care must be taken when using the system not to inadvertantly select FEATHER.

Superfine

A flight fine pitch stop prevents the blade angle decreasing below 22’ in flight. The stops can be removed on the ground to allow the blades to go into a finer angle, known as superfine, which provides a braking effect on landing, and prevents excess torque when taxying.  When the Superfine Master switch is set to ON, and the undercarriage weight switches have closed on the ground, when the throttles are closed the fine pitch stop bolts withdraw, and the propeller will move into the superfine range.

At the foot of the main panel there are four amber Superfine Warning lamps which illuminate when the propeller is at 18’ or below, and an amber Armed lamp which has two functions; it will light when the superfine master is selected on, or the reverse system is armed by pulling the Reverse lever rearward.

Superfine must be cancelled before takeoff, to ensure the fine pitch stop bolts have engaged, and the propellers must also be returned to flight fine pitch before shutting the engines down.  Four superfine isolation switches appear at the top of the panel when the propeller controller window is selected, and are used when deselecting superfine before closing down the engines.(See Engine and Flight Handling).

Reverse

The propellers can be moved into a reverse angle of -18’ for additional braking, this being accomplished by moving the Reverse Arming Lever rearward to the REVERSE position, thus enabling the throttles to be retracted beyond the idle position initially to the brake dwell setting, then further if an increase in reverse power is required. Clicking on the BRAKE DWELL text on the quadrant will select all propellers to brake dwell, with further power available by retarding each throttle or using the keyboard or joystick controls.

Four green Propeller Reversing warning lamps illuminate when the blades are reversing, the superfine lamps will also light. The single Armed light will also come on when reverse is armed.

The propellers should be brought out of reverse before the end of the landing run, ensuring superfine is on, then the actuators run to MIN. The reverse system may be left armed while taxying as brake dwell may be required to avoid overheating of the wheel brakes.

Feathering

A combined engine fire warning and feathering button is provided for each engine on the centre panel. The light will come on to warn of a fire or if a propeller is feathered. The button is depressed to feather or unfeather a propeller. Additionally, the propeller standby control can be used for feathering. (See Engine and Flight Handling).

Britannia 253

Starting Controls

The engine starting controls are grouped on the left overhead panel which is opened with the  icon. The normal starting order is 3,4 2,1.

The ground starting controls consist of a start master switch, engine selector, starter button end green cycle warning lamp. Additionally there are flight starting controls, which are also required for ground starting; four starter fuel pumps which supply additional fuel to the engine during start and are sprung to the centre off position, four three position igniter plug switches, which should be selected to NORMAL for ground starting or RE-IGNITE for relighting in the air, and four high pressure fuel cock switches. The HP cock switches are mechanically linked one way with the propeller brake switches to prevent the engines being started with the prop brakes applied. A magnetic indicator shows ON or OFF for each prop brake or cross-hatch when there is no electrical supply. There are a second set of HP cock switches on the centre overhead panel for emergency use and both sets perform the same function, closing the fuel supply to the engine.

Emergency Controls

Each engine has identical emergency controls, divided by black and yellow flanges. The controls consist of a fire warning lamp, HP fuel cock switch and magnetic indicator, LP fuel cock switch and magnetic indicator, oil cock switch and magnetic indicator, a fire door operating switch and dual shot fire extinguisher buttons guarded by red covers. FS does not yet support extinguishing systems, therefore the buttons are inoperative. The oil cock switches are linked one way with the LP cocks so that they cannot be closed without shutting down the engine. The alarm switches can be used to cancel a fire warning.

Britannia 253

Water Injection

To provide increased power at high ambient temperatures without exceeding JPT limitations a water injection system is provided. By cooling the temperatures throughout the engine a higher RPM can be achieved. A 12 gallon water tank behind each nacelle supplies water which is injected into the engine air intakes. The system is armed when the master switches at the forward end of the quadrant are selected on. Water flow commences when the throttles are opened beyond 11000 CRPM. A green Water Flow warning lamp illuminates when the injection starts and will remain on until the water is exhausted. The supply lasts approximately 5 ¾  minutes. The water flow system increases the maximum CRPM value from 11775 CRPM to 12000 CRPM.

At ambient temperatures in excess of 35’C, 10 CRPM must be deducted from the maximum setting for each 1’C rise in temperature.

Instrumentation

Each engine has :

A Torquemeter and adjacent water flow lamp.

A  dual scale Compressor RPM gauge. The outer scale shows 0-12300 RPM, the inner sensitive scale shows 11300 RPM – 12300 RPM

A Jet Pipe Temperature gauge.

A Propeller RPM gauge. The PRPM gauges are accessed by clicking the Test Main Legs button to the right of the undercarriage position indicator.

Additionally there are dual oil pressure gauges for Nos 1 and 2 engines and Nos 3 and 4 engines. The oil temperature gauge shows Nos1 and 2 engine readings by default, but can be clicked to show Nos 3 and 4.

All of the engine instruments are tooltipped for clarity.

Britannia 253

3. FUEL SYSTEM

Fuel Tanks

The fuel system consists of four main tanks and three transfer tanks which feed the main tanks. In order for the system to function correctly, the tanks have been renamed in the aircraft cfg file, and will appear on the fuel menu as follows:

| FUEL TANK | FS EQUIV | IMP GALLS | US GALLS | LBS |
| --- | --- | --- | --- | --- |
| Port Transfer | * | 460 | 552 | 3680 |
| No1 Port | Left Aux | 1545 | 1855 | 12365 |
| No2 Port | Left Tip | 1670 | 2004 | 13365 |
| Centre | Centre | 1230 | 1477 | 9846 |
| No3 Stbd | Right Tip | 1670 | 2004 | 13365 |
| No4 Stbd | Right Aux | 1545 | 1855 | 12365 |
| Stbd Transfer | * | 460 | 552 | 3680 |

*Note that any quantity over 1855 usg loaded into the left or right aux tanks on the menu will be placed in the Port or Stbd Transfer Tanks respectively. As fuel is used from No1 and No3 tanks, the fuel from the transfer tanks will be pumped in to those tanks until the transfer tanks are empty. The transfer tank pump switches are permanently on.

The four main tanks are connected by three crossfeed cocks and any tank or combination of tanks can supply the engines.

The Centre Transfer Tank can supply Nos 2 and 3 Tanks, however this tank should only be used if the others are full. To transfer from the centre tank, the booster pumps for the recipient tank(s) 2 and/or 3 should be on, the centre transfer pumps switched on and the transfer cocks opened, checking the magnetic indicators for continuity.

Controls

The controls for the fuel system are on the engineer’s panel and are opened with the  icon.

Britannia 253

The panel is a schematic diagram with white lines connecting the controls and gauges and three position magnetic indicators representing the fuel cocks. The indicators show continuity or discontinuity or crosshatch when de-energised, except for the centre transfer cock indicators which show black.

Two booster pumps in each tank, at least one of which must be on if the tank is in use, raise the fuel pressure for delivery to the engine. An amber warning lamp shows if the pump fails when switched on.

The fuel leaves the tank via a tank isolation cock, normally used for defuelling and by engineering, and these are permanently open. The fuel passes to a common delivery pipe divided into four sections by the three crossfeed cocks. It then passes through the LP fuel cocks and flowmeter before being introduced to the engine at the HP fuel cock, their controls being on the overhead emergency panel.

All of the gauges are calibrated in pounds. The flowmeters incorporate a counter which shows pounds used, and is, in fact, a division of the total fuel used. Clicking on any of the gauges will reset them all.

There are four LP Fuel amber warning lamps on the centre instrument panel.

Normal Use of the System

Ideally, the four main tanks should supply their respective engines, and this must be used for takeoff and landing (default). As early as is practical during the climb any imbalance between the tanks should be eliminated by running all engines from the outer or inner tanks as required, until the main tanks contain similar amounts of fuel. To balance the tanks:

Open the Port and Starboard Crossfeed Cocks, and check the MI’s.

Switch off both Booster Pumps in the tanks not in use.

When balance is achieved:

Switch on the Booster Pumps in all four tanks.

Close the Crossfeed Cocks and check the MI’s.

The Centre Crossfeed cock should only be opened if both tanks on a particular side are out of use.

Fuel Jettisoning

Flight Simulator does not support fuel jettisoning, however the gauges are arranged such that operation of the jettison switches will appear to dump fuel, although the actual quantity remains unaltered. The switches are guarded by cross-hatched covers and may only be operated once during flight.

To jettison fuel:

Switch the jettison switches on for the tanks to be reduced.

When the required level is reached switch off the jettison switches.

Britannia 253

4. FLYING CONTROLS, FLAPS and UNDERCARRIAGE

Flying Controls

The primary control surfaces are operated by the effect of trailing edge servo tabs, there being no direct linkage between the cockpit controls and primary surfaces.

Trim controls for the ailerons and rudder are located on the pedestal and both have an associated indicator.

The controls are locked by default, requiring the lever on the quadrant to be moved forward to the unlock position to free the controls. A red LOCKED warning lamp for each surface is on the coaming panel; A E R E A.

Flaps

The flaps are electrically operated via a lever on the right of the pedestal and have four preset positions; UP, 15’, 30’ and 45’ down. A flap position indicator is between the flight and engine instruments on the main panel.

Maximum speeds associated with the flaps are:

UP – 15’ 210 kt

15’ – 30’ 180 kt

30’ – 45’ 160 kt

Undercarriage

The undercarriage is hydraulically operated and comprises two main leg units of four wheel bogie layout and a twin wheel nose leg.

The position indicator on the main panel gives the following indications:

Green lights : Undercarriage locked down

Red lights: Undercarriage unlocked or in transit.

No lights: Undercarriage locked up

Additionally the nose red lamp will glow of the flap lever is selected beyond the 15’ position and any undercarriage leg is not locked down, or if any throttle is retarded below about ¼ open and any undercarriage leg in not locked down. FS also provides us with an aural warning if the flaps are extended beyond 20’ before undercarriage extension.

Maximum speed for u/c extension 180kt IAS

Maximum speed with u/c extended 200kt IAS

The undercarriage position indicator may be clicked to cycle the undercarriage, there being no physical lever included on the panel.

Britannia 253

5. DE-ICING SYSTEMS

The controls for the de-icing equipment are grouped on the right of the overhead panel which is opened with the  icon. There are separate systems for wing de-icing, tail de-icing, engine intake de-icing, compressor guide vane heating and propeller deicing, as well as protection for the pressure heads.

Weight switches inhibit the systems on the ground except for the propeller de-icing and guide vane and pitot heat.

Wing De-icing

The wings are de-iced thermally using bleed air from all four compressors, which is passed through heat exchangers before being circulated through a double skin in the leading edges and exhausted under the nacelles and wingtips.

The system is controlled by a Wing De-Ice Master switch, which has two positions, ICING and OFF. Additionally there are four individual temperature control switches which are operative when the system is on. And are used to maintain the desirable skin temperature of approximately 70’C.

Britannia 253

Four linear indicators, show the port and starboard inboard and outboard skin temperatures. The four magnetic indicators show ON with the system functioning normally,

or HIGH when the temperature reaches approximately 90’C. they show cross-hatch with the system off or when de-energised. A red system failure warning lamp is adjacent to the master switch. With the system on and functioning, the FS airframe de-icing is activated.

The system should not be switched on until airborne, and used only in icing conditions.

Tail Unit De-icing

The tail is de-iced by electrically heated mats, some of which are heated continuously and others cyclically. The cyclically heated areas are operated by one of two cyclic timers which act as standbys for each other.

The Tail De-icing Master switch has three positions; ABOVE -12’C, OFF and BELOW -12’C. The two rotary switches are used in conjunction with the Timer Changeover switch

to select the desired timer to the rate of cycling, and a desynn indicator rotates confirming the cyclic operation. A tail system failure red warning light and magnetic indicator provide indication of a failure with the system on. The indicator shows which part of the system has failed, or in the case of both parts failing, that which failed last. The indicator normally shows black, or either CYC or LE in case if failure.

The system should be used in icing conditions and after leaving them, but not switched on above 5’C.

The elevator horn balance is de-iced continuously by heated mats controlled by a single switch. A Horn Failure warning lamp lights if the power supply fails.

Compressor Guide Vane De-Icing

Hot air is passed through the hollow guide vanes at the entry to the compressor, Proteus engines being susceptible to this type of icing. The system is on Auto by default, and would be selected on and off with ice detector operation. The system would be selected to manual after starting engines and during takeoff in any conditions of precipitation or icing,

or before entering cloud and left on for 15 minutes after leaving cloud. The controls are for procedure only and have no effect on FS.

Propeller De-Icing

The propellers are electrically de-iced by external and internal heating elements respectively. The 200VAC supply is from No1 Alternator. Loads can be transferred as described in the Electrical System. Each propeller is cyclically heated in turn, and control is via a single Prop De-Ice Master switch which has three positions; SHORT CYCLE, OFF and LONG CYCLE. When the short cycle is selected each propeller in turn receives current for periods of 30 seconds,and when long cycle is selected, for periods of two minutes.

A magnetic indicator for each propeller shows cross-hatch with the system off or ON or OFF with the system on, in accordance with the cycle.

Britannia 253

Although available on the ground, propeller de-icing should not be switched on while taxying unless absolutely essential. The engine supplying the system, normally No1, should be run at a minimum of 760 PRPM. In flight the system should be on as soon as icing is encountered. The system should not be switched on above 10’C and the long cycle should be used at OATs below -15’C . The system has been arranged so that any feathered propellers are automatically taken out of the cycle. When the system is operating, FS Prop De-ice is activated.

Engine Intake De-Icing

The outer surface of each intake is heater internally by hot gas which is then exhausted. The nose pieces separating the individual ducts are de-iced electrically both continuously and cyclically.

The system is controlled by a single Engine De-Ice Master switch which has two positions; ICING and OFF. The heat could be controlled in a similar way to that of the wing ducts, however for FS purposes the heat switches should be left on AUTO. Four magnetic indicators show ON or OFF if the system is functioning, and cross-hatch when de-energised. Cowl heat has been proven largely ineffective at speeds in excess of 200kt, and outside the temperature range +2 to -12 ‘C, therefore the system should be switched off in these conditions.

Pressure Head De-Icing

The pressure heads and static vents are de-iced electrically, a single control switch and magnetic indicator beind added to the overhead for convenience. The indicator show ON with the system functioning and cross-hatch at other times.

The heaters must not be switched on for more than 10 minutes prior to takeoff of the OAT is below 4’C, or 5 minutes if the OAT is greater.

6. MINOR SYSTEMS

Lights

The controls for the external lights open over the oil pressure gauges by clicking on either gauge. The internal panel lights are controlled by the rightmost of the six rheostats at the bottom of the main panel.

Door Warning Lamp

A single red door warning lamp on the coaming panel can be clicked to cycle the doors.

Data Card & Automated Checklist

An automated checklist can be opened with the  icon. The Speed and Weight Card is opened with the  icon.

Rapid Starting

The engine start sequence may be performed automatically by clicking the  icon, providing all CRPMs are below 1100.

Britannia 253

7. FLIGHT SYSTEM, AUTOPILOT & RADIOS

Flight System

Radio Selector.

The radio selector is used to select Nav1 or Nav2 information to the Flight System. The top switch is selected to 1 or 2 as desired. Note that the Azimuth Director and Pitch Director will only respond to Nav1 displacements so if Nav2 is selected the radial and glide path must be flown raw. When the switch is Off, the Azimuth Director will respond to differences between the Heading Pointer and Heading Index on the Beam Compass.  A magnetic indicator shows the input to the Flight System.

Director Horizon

- Beam Flag. Visible when a steady VOR/Localiser signal is received

- G/P Flag. Visible when a steady Glide Path signal is received. Pitch scale must be

wound fully down for GP signals.

- Pitch Director. Shows demand for pitch corrections relative to GP deviation.

- Horizon Bar. Moves relative to aircraft bank.

- Pitch Pointer. Moves vertically relative to aircraft pitch

- Pitch Scale.

- Bank Scale.

- Azimuth Director. Shows demand for heading and bank corrections.

- Ring Sight. Placing the Ring Sight over the Azimuth Director will correctly bank the aircraft.

- Pitch Scale Setting Knob.

Beam Compass

- Sense Switch. Points up when flying towards a station and down when

flying from it. With the switch across, no signals are received.

- Compass Scale. At its top datum, the Compass Scale shows the VOR or ILS course selected. It may be rotated using the area over the Compass Warning Light.

- Heading Pointer. Shows aircraft heading relative to the Compass Scale.

- Heading Index. Used to set desired heading

- Beam Displacement Bar. Shows aircraft deviation from the selected course.

- Beam Displacement Scale.

- Setting Knob. Used to alter the Heading Index.

- Compass Warning Lamp (Inoperative).

Britannia 253

Omni Bearing Indicator

The OBI shows Nav2 information by default, clicking on the face of the instrument will change over to Nav1.

Radio Magnetic Indicator

A twin pointer RMI is fitted, the red single pointer can show either VOR1 or ADF1, the green dual pointer VOR2 or ADF2. Switching is above the CDI on the main panel and two magnetic indicators adjacent to the RMI show the selection which has been made if a signal is received. Note the MI will show black if VOR is selected and an ILS frequency has been tuned, and the signal cannot be displayed on the RMI.

Autopilot Controller

The autopilot controller is on the pedestal opened with the  icon.  Before the autopilot can be used, the Power Master Switch on the pedestal should be switched on.

- Power Switch. The Power switch must be pulled to initiate the supply to the Autopilot. This is effectively the FS Autopilot master. When the AP is ready for use the amber Ready lamp will illuminate.

- Engage Button is pressed to engage the AP and hold the current pitch attitude.

- Channel Switches . May be used to isolate the Rudder, Elevator or Aileron channels from the Autopilot.

- Height/Airspeed Lock. Turned to engage the height or airspeed lock.

- Heading Hold Button. When engaged the aircraft may be steered by altering the Heading Index on the Beam Compass or with the turn knob on the AP Controller.

- Beam Coupling Switch. Pull to engage the FS Nav1 hold function.

- Glide Coupling Switch. Pulled to engage FS Approach Hold function. This will also cause the Beam Switch to engage.

- Pitch Switch.  Used to vary the nose up or down pitch after pressing the green Engage button.

- Turn Knob. Used to accomplish manual turns using the autopilot. Beam/Glide and

Heading functions must be off.

- Ready Lamp. Illuminates after about 30 seconds from pulling the power switch On. The lamp will extinguish when any function is engaged.

Note that the Autopilot will only respond to Nav1 information and that once the Power Switch is depressed, all Pitch/Azimuth Director indications are relative to AP commands.

Engage & Trim Indicator

The Engage and Trim Indicator shows elevator trim movements on the centre scale. Additionally three flags marked IN will appear when the relevant control surface channel is engaged.

Radio Installation

The Radio panel is opened with the  icon, and dual VOR/ILS  and Comm VHF radios are fitted in addition to dual ADF sets.  A DME indicator is on the main panel and has to outputs; the counter scale indicates DME2, the pointer scale indicates 1. By default the pointer shows nautical miles x 10 up to a maximum of 140, however clicking on the top half if the dial selects nautical miles up to a maximum of 14. A red bar covers the DME2 counter if there is no signal and a black and yellow cross-hatched flag appears at the top of the instrument if there is no DME1 signal.

The TACAN instrument has DME1 information displayed in the centre, and the pointer shows the aircraft bearing to/from the station.

Britannia 253

8. LIMITATIONS

| AIRFRAME LIMITATIONS |  |
| --- | --- |
| NEVER EXCEED SPEED Vne | 288 KT or 0.65 MACH |
| NORMAL OPERATING SPEED LIMIT Vno | 258 KT or 0.60 MACH |
| AUTOPILOT MAX OPERATING SPEED | 258 KT or 0.60 MACH |
| MANOEUVRING SPEED, FULL AILERON/RUDDER | 203 KT |
| FLAPS UP – 15’ | 210KT |
| 15’ – 30’ | 180 KT |
| 30’ – 45’ | 160 KT |
| UNDERCARRIAGE EXTENSION | 180 KT |
| WITH U/C EXTENDED | 200 KT |
| LANDING LAMPS EXTENSION | 165 KT |
| EXTENDED | 210 KT |
| CROSSWIND MAX FOR TAKEOFF | 20 KT |
| MAX FOR LANDING | 25 KT |
| FUEL JETTISONING MAX SPEED | 210 KT |
| MAXIMUM TAKEOFF WEIGHT | 185000 LB |
| MAXIMUM LANDING WEIGHT | 140000 LB |
| POWER PLANT LIMITATIONS |  |
| OIL TEMPERATURE MAX FOR STARTING | 35’C |
| MIN FOR OPENING UP | 5’C |
| MAX IN FLIGHT | 110’C |
| MAX BEFORE TAKEOFF | 90’C |
| OIL PRESSURE NORMAL | 70 - 80 PSI |
| MIMIMUM ABOVE 10000 CRPM | 50 PSI |
| PROPELLER LIMITATIONS |  |
| PROHIBITED FOR CONTINUOUS | 360 – 560 PRPM |
| INFLIGHT MINIMUM | 850 PRPM |
| MAX CONTINUOUS | 920 PRPM |
| MAXIMUM PERMISSIBLE | 1000 PRPM |
| ENGINE LIMITATIONS MAX DRY | 11775 CRPM 815 PSI |
| MAX WET | 12000 CRPM 815 PSI |
| MAX CONT BELOW FL150 | 11500 CRPM 780 PSI |
| MAX CONT ABOVE FL 150 | 11650 CRPM 780 PSI |
| MIN FLIGHT IDLING 7900 CRPM @ 850 – 920 PRPM |  |
| MAX REVERSE | 11500 CRPM for 1 MIN |

Britannia 253

9. ENGINE & FLIGHT HANDLING

Starting

The engines are normally started in the order 3,4,2,1. It is assumed that the engines have been previously shut down correctly and that the Before Start checks have been completed.

- Prop Controller…………………………………………..…MIN, CHECK MI’s

- Prop Master…………………………………………………………………OFF

- Superfine Master…………………………………………………CHECK OFF

- Superfine Isolation Switches……………………………....ALL AT ISOLATE

- Throttles………………………………………………………………….…IDLE

- Reverse Arming lever………………………………….………….FORWARD

- Prop Lights……………………………………………………..……..ALL OUT

- Gust Locks…………………………………………………………………….IN

- Prop Brakes…………………………………………………………………OFF

- HP Cocks…………………………………………………SHUT, CHECK MI’s

- Crossfeed Cocks & Transfer Cocks………………………………..…..SHUT

- Booster Pumps………………………MAIN TANKS ALL ON, LIGHTS OUT

- LP Fuel Lights………………………………………………………………OUT

- Start Selector………………………………………………………………NO.3

- Start Master……………………………………………………………..…...ON

- Igniter Switch…………………………………………………..No3 TO NORM

- Starter Button……………………………………………………………PRESS

- HP Cock……………………………………………………….........No3 OPEN

- 1000-1200 RPM…………………..…OPERATE FUEL PUMP FOR ½ SEC

- Engine RPM and JPT Stable………………..….SUPERFINE MASTER ON

No3 SUPERFINE ISOL SWITCH NORM

ARMED & No3 SUPERFINE LIGHT ON

CHECK PRPM RISES to 560

IGNITER SWITCH OFF

Repeat items 14 -20 for remaining engines.

21.Start Master………………………………………………………………….OFF

22.Start Selector ………………………………………………………………..No3

23.MV & LV Master Switches……………..…FLIGHT, GROUND LIGHTS OUT

24.Main Power Fail Light……………………………………………………....OUT

25.Ammeters/Voltmeters……………………………………………….….CHECK

26.Inverters………………………………………………………………….ALL ON

27.Prop De Ice…………………………………………………………...AS REQ’D

28.Guide Vane Heat………………………………………………………….AUTO

The engines should be stable at 6500-7300 CRPM, and the aircraft will move off with little or no increase in power required, once under way the throttles should be adjusted to idle at 7000 CRPM. Minimum braking should be used when taxying, and if necessary a pair of engines may be set to brake dwell.

Complete the Taxy Checks.

Britannia 253

Holding Point

Before lining up on the runway the engines must be brought out of superfine and a check made that the fine pitch stop bolts have engaged. This takes a bit of getting used to as it is designed to keep generated power on line.

1.Brakes…………………………………………………..……………………….….ON

2.Reverse Arming Lever……..…………………………………………….REVERSE

3.Nos 2 & 3 Engines………………………………………SELECT BRAKE DWELL

4.Nos 1 & 4 Engines……OPEN THROTTLES UNTIL OUTBOARD SUPERFINE

LIGHTS OUT, SELECT SUPERFINE MASTER OFF

RUN ALL PROP ACTUATORS TO MAX

5.Nos 1 & 4 Engines…….RETURN TO IDLE, CHECK MAIN POWER LIGHT ON

& SUPERFINE LIGHTS REMAIN OUT.

THEN RETURN TO 9000CRPM & CHECK MAIN

POWER LIGHT GOES OUT

6.Nos 2 &3 Engines……..RETURN TO IDLE, CHECK MAIN POWER LIGHT ON

& SUPERFINE & REVERSING LIGHTS OUT

THEN ALIGN WITH OUTBOARDS AT 9000 CRPM &

CHECK MAIN POWER LIGHT OUT

7.Reverse Arming Lever……………..……..FORWARD, ALL PROP LIGHTS OUT

The throttles may be returned to idle if departure is imminent, however the aircraft will revert to battery power.

Complete the Before Takeoff checks. Take Off Data appears on a data card selected with the V icon.

Take Off

If water injection is used, the system should be armed when lined up for takeoff. Power should be advanced initially to 11000 CRPM, before applying takeoff power, which is 11775 CRPM unless ambient conditions dictate otherwise. At ambient temperatures in excess of 35’C, 10 CRPM must be deducted from the maximum setting for each 1’C rise in temperature. If water injection is in use, check the lights come on, max CRPM 12000.

Once the undercarriage and flaps have been retracted, set maximum continuous power, 11500 CRPM, 780 PSI, whichever is lower, and 920 PRPM. The speed should be allowed to build up to 180kt, until rate of climb drops below 1000fpm.

When the water flow lights go out, JPT will rise slightly, by approximately 20’, then switch the system off. The throttles may require slight adjustment.

Complete the After Takeoff Checks.

Cruise

Above 15000 ft, CRPM may be increased slightly to 11650, observing the limiting torque

figure. Operational data recommends 860 PRPM in the cruise, although max continuous may be required at higher weights. Conversely at lower altitudes and/or lighter weights the engines should be throttled to give an IAS not in excess of Vno, 258kt.

Britannia 253

Descent

Prior to beginning the descent, reduce power to flight idle, approximately 8000 CRPM

But not less than 135 PSI torque. Do not close the throttle to the stop. As airspeed reduces, begin the descent and perform the necessary checks. Ensure that the fuel tanks are not excessively out of balance.

Holding

If the aircraft is to be flown level at any point during the descent, 200-225 PSI torque gives an airspeed of around 200kt, if speed restrictions necessitate, reduce power to 190-200 PSI. With flap 15’ and undercarriage extended, around 300 PSI may be required for level flight.

Approach

Complete the approach checks, ensuring that PRPM is increased to 920 before commencing the final. Reduce speed below 210 kt and set 15’ flap. It is desirable to be level at 2000-2500 ft on the QNH and at an airspeed of 170-180 kt when commencing the final at ILS intercept.

Finals

Select undercarriage down below 180 kt, as the glide path pointer on the CDI begins to approach the centre of the instrument, following the pitch pointer on the director horizon, but avoiding chasing the instrument. Do not descend below the glide path. 200-250 PSI torque is normally sufficient to keep the aircraft on a steady descent, depending on weight.  At about 5 DME select flap 30’, a slight pitch down is experienced, and the airspeed will begin to reduce. An increase in power may be required to maintain 130-140kt until 400ft aal, then select  flap 45’ and reduce speed to achieve Vat crossing the threshold.

After Landing

Once the nosewheel is on the ground, select the superfine master on, and observe the superfine lights illuminate. If reverse is required, move the arming lever to reverse, select

brake dwell, and increase reverse power if necessary.  Towards the end of the landing run

select MIN on the all props switch, and cancel reverse, returning the lever to forward.

Once clear of the runway, during the after landing checks, the aircraft is normally taxied on the inboard engines only. Before closing down the outboard engines, switch off alternators 1 & 4, increase power on the outboards, until their superfine lights go out, then switch Nos 1 & 4 superfine isolate switches to Isolate. Return the outboard throttles to idle and observe their superfine lights stay out. Close the outboard HP Cocks to shut down the engines.

Closing Down

On stand, before closing down the remaining engines, increase power until the inboard superfine lights cancel, switch off the superfine master and select Isolate on the inboard superfine isolate switches. Close the HP Cocks and complete the remaining checks.

Britannia 253

Emergency Procedures

Manual Feathering

Should the need arise to shut down an engine in flight:

- Throttle.........................................................................CLOSE

- Feathering Button...................................................PRESS IN

- HP Cock........................................................................CLOSE

- Alternator…………………………...…OFF, TRANSFER LOAD

Engine Fire Drill

- Engine……………………………………………………IDENTIFY

- Throttle…………………………………………………...…CLOSE

- Feather Button………………………………………….PRESS IN

- HP Cock…………………………………………………….CLOSE

- LP Cock…………………………………………………….CLOSE

- Oil Cock…………………………………………………….CLOSE

- Fire Door…………………………………………………...CLOSE

- Fire Extinguisher……………………………………...OPERATE

- Alternator………………………………OFF, TRANSFER LOAD

Unfeathering and Relighting

Relighting in the air should be successful at any normal speed, however 160-200kt is optimal.

- Throttle .....................................................................................IDLE

- Booster Pumps....................................................................ALL ON

- HP Cock…………………………………………………………....OPEN

- Feathering Button....................................PRESS TO UNFEATHER

- Igniter Switch...................................................................RE-IGNITE

- Starter Fuel Pump................................................HOLD ON, ½ SEC

- PRPM………………………………………………………………RISING

- Alternator...................RESET,THEN ON, LOAD TRANS TO NORM

- Throttle.................................................OPEN TO MATCH OTHERS

- Igniter Switch………………………………………………………...OFF

- Relight Fails………….. HP COCK SHUT,  3 MINUTES BETWEEN

ATTEMPTS

The information contained in this manual is based on Britannia C. Mk1/ 253 data, the information is for Flight Simulation use only and should not be considered for use with the real aircraft.

All brand names used throughout remain copyrights of their owners and are used as reference only.

Fraser McKay, April 2007